# 3.6.1 Packet Tracer - Implement VLANs and Trunking

Addressing Table

| Device | Interface | IP Address | Subnet Mask | Switchport | VLAN |
| --- | --- | --- | --- | --- | --- |
| PC1 | NIC | 192.168.10.10 | 255.255.255.0 | SWB F0/1 | VLAN 10 |
| PC2 | NIC | 192.168.20.20 | 255.255.255.0 | SWB F0/2 | VLAN 20 |
| PC3 | NIC | 192.168.30.30 | 255.255.255.0 | SWB F0/3 | VLAN 30 |
| PC4 | NIC | 192.168.10.11 | 255.255.255.0 | SWC F0/1 | VLAN 10 |
| PC5 | NIC | 192.168.20.21 | 255.255.255.0 | SWC F0/2 | VLAN 20 |
| PC6 | NIC | 192.168.30.31 | 255.255.255.0 | SWC F0/3 | VLAN 30 |
| PC7 | NIC | 192.168.10.12 | 255.255.255.0 | SWC F0/4 | VLAN 10 VLAN 40 (Voice) |
| SWA | SVI | 192.168.99.252 | 255.255.255.0 | N/A | VLAN 99 |
| SWB | SVI | 192.168.99.253 | 255.255.255.0 | N/A | VLAN 99 |
| SWC | SVI | 192.168.99.254 | 255.255.255.0 | N/A | VLAN 99 |

Objectives

- Part 1: Configure VLANs

- Part 2: Assign Ports to VLANs

- Part 3: Configure Static Trunking

- Part 4: Configure Dynamic Trunking

Background

You are working in a company that is getting ready to deploy a set of new 2960 switches in a branch office. You are working in the lab to test out the VLAN and trunking configurations that are planned. Configure and test the VLANs and trunks.

Instructions

## Part 1: Configure VLANs

Configure VLANs on all three switches. Refer to the VLAN Table. Note that the VLAN names must match the values in the table exactly.

VLAN Table

| VLAN Number | VLAN Name |
| --- | --- |
| 10 | Admin |
| 20 | Accounts |
| 30 | HR |
| 40 | Voice |
| 99 | Management |
| 100 | Native |

| SWA | SWA>enable SWA#configure terminal SWA(config)#vlan 10 SWA(config-vlan)#name Admin SWA(config-vlan)#vlan 20 SWA(config-vlan)#name Accounts SWA(config-vlan)#vlan 30 SWA(config-vlan)#name HR SWA(config-vlan)#vlan 40 SWA(config-vlan)#name Voice SWA(config-vlan)#vlan 99 SWA(config-vlan)#name Management SWA(config-vlan)#vlan 100 SWA(config-vlan)#name Native |
| --- | --- |
| SWB | SWB>enable SWB#configure terminal SWB(config)#vlan 10 SWB(config-vlan)#name Admin SWB(config-vlan)#vlan 20 SWB(config-vlan)#name Accounts SWB(config-vlan)#vlan 30 SWB(config-vlan)#name HR SWB(config-vlan)#vlan 40 SWB(config-vlan)#name Voice SWB(config-vlan)#vlan 99 SWB(config-vlan)#name Management SWB(config-vlan)#vlan 100 SWB(config-vlan)#name Native |
| SWC | SWC>enable SWC#configure terminal SWC(config)#vlan 10 SWC(config-vlan)#name Admin SWC(config-vlan)#vlan 20 SWC(config-vlan)#name Accounts SWC(config-vlan)#vlan 30 SWC(config-vlan)#name HR SWC(config-vlan)#vlan 40 SWC(config-vlan)#name Voice SWC(config-vlan)#vlan 99 SWC(config-vlan)#name Management SWC(config-vlan)#vlan 100 SWC(config-vlan)#name Native |

## Part 2: Assign Ports to VLANs

### Step 1: Assign access ports to VLANs

On SWB and SWC, assign ports to the VLANs. Refer to the Addressing Table.

| SWB | SWB(config)#int f 0/1 SWB(config-if)#switchport mode access SWB(config-if)#switchport access vlan 10 SWB(config-if)#exit SWB(config)#int f 0/2 SWB(config-if)#switchport mode access SWB(config-if)#switchport access vlan 20 SWB(config-if)#exit SWB(config)#int f 0/3 SWB(config-if)#switchport mode access SWB(config-if)#switchport access vlan 30 |
| --- | --- |
| SWC | SWC(config)#int f 0/1 SWC(config-if)#switchport mode access SWC(config-if)#switchport access vlan 10 SWC(config-if)#exit SWC(config)#int f 0/2 SWC(config-if)#switchport mode access SWC(config-if)#switchport access vlan 20 SWC(config-if)#exit SWC(config)#int f 0/3 SWC(config-if)#switchport mode access SWC(config-if)#switchport access vlan 30 SWC(config-if)#exit SWC(config)#int f 0/4 SWC(config-if)#switchport mode access SWC(config-if)#switchport access vlan 10 |

### Step 2: Configure the Voice VLAN port

Configure the appropriate port on switch SWC for voice VLAN functionality.

SWC(config)#int f 0/4 SWC(config-if)#switchport voice vlan 40 SWC(config-if)#mls qos trust cos

### Step 3: Configure the virtual management interfaces

- Create the virtual management interfaces, on all three switches.

- Address the virtual management interfaces according to the Addressing Table.

- The switches should not be able to ping each other.

| SWA | SWA(config)#int vlan 99 SWA(config-if)#ip address 192.168.99.252 255.255.255.0 SWA(config-if)#no shutdown |
| --- | --- |
| SWB | SWB(config)#int vlan 99 SWB(config-if)#ip address 192.168.99.253 255.255.255.0 SWB(config-if)#no shutdown |
| SWC | SWC(config)#int vlan 99 SWC(config-if)#ip address 192.168.99.254 255.255.255.0 SWC(config-if)#no shutdown |

## Part 3: Configure Static Trunking

- Configure the link between SWA and SWB as a static trunk. Disable dynamic trunking on this port.

| SWA | SWA(config-if)#int g 0/1 SWA(config-if)#switchport mode trunk |
| --- | --- |
| SWB | SWA(config-if)#int g 0/1 SWB(config-if)#switchport mode trunk |

- Disable DTP on the switch port on both ends of the trunk link.

| SWA | SWA(config-if)#switchport nonegotiate |
| --- | --- |
| SWB | SWB(config-if)#switchport nonegotiate |

- Configure the trunk with the native VLAN and eliminate native VLAN conflicts if any.

| SWA | SWA(config-if)#switchport trunk native vlan 100 |
| --- | --- |
| SWB | SWB(config-if)#switchport trunk native vlan 100 |

## Part 4: Configure Dynamic Trunking

- Assume that the trunk port on SWC is set to the default DTP mode for 2960 switches. Configure G0/2 on SWA so that it successfully negotiates trunking with SWC.

SWA(config)#int g 0/2 SWA(config-if)#switchport mode dynamic desirable

- Configure the trunk with the native VLAN and eliminate native VLAN conflicts if any.

SWC(config)#int g 0/2 SWC(config-if)#switchport mode trunk SWC(config-if)#switchport trunk native vlan 100