# 4.4.8 Packet Tracer - Troubleshoot Inter-VLAN Routing

Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway | VLAN |
| --- | --- | --- | --- | --- | --- |
| R1 | G0/1.10 | 172.17.10.1 | 255.255.255.0 | N/A | VLAN 10 |
|  | G0/1.30 | 172.17.30.1 | 255.255.255.0 | N/A | VLAN 30 |
| PC1 | NIC | 172.17.10.10 | 255.255.255.0 | 172.17.10.1 | VLAN 10 |
| PC3 | NIC | 172.17.30.10 | 255.255.255.0 | 172.17.30.1 | VLAN 30 |

Objectives

- Part 1: Locate Network Problems

- Part 2: Implement the Solution

- Part 3: Verify Network Connectivity

Scenario

In this activity, you will troubleshoot connectivity problems caused by improper configurations related to VLANs and inter-VLAN routing.

Instructions

## Part 1: Locate the Network Problems

Examine the network and locate the source of any connectivity issues.

Commands you may find useful include:

R1# show ip interface brief R1# show interface g0/1.10 R1# show interface g0/1.30 S1# show interface trunk

- Test connectivity and use the necessary show commands to verify configurations.

- Verify that all configured settings match the requirements shown in the Addressing Table.

- List all of the problems and possible solutions in the Documentation Table.

Documentation Table

| Problems | Solutions |
| --- | --- |
| The PC3 default gateway is configured incorrectly | Change the default gateway on PC3 from 172.17.10.1 to 172.17.30.1 |
| Interface G0/1 on S1 is configured as access instead of trunk | Configure the interface G0/1 on S1 as a trunk use the “switchport mode trunk” command |
| Subinterface G0/1.10 is administratively down | Give the “no shutdown” command for the subinterface G0/1.10 |
| Subinterface G0/1.10 is configured incorrect VLAN | Remove the incorrect Encapsulation dot1q, and assign the correct Encapsulation dot1q 10, then reassign the ip address for Subinterface G0/1.10. |
| Subinterface G0/1.30 is configured incorrect VLAN | Remove the incorrect Encapsulation dot1q, and assign the correct Encapsulation dot1q 30, then reassign the ip address for Subinterface G0/1.30 |

## Part 2: Implement the Solutions

Implement your recommended solutions.

| 1) | Change the default gateway on PC3 from 172.17.10.1 to 172.17.30.1 |
| --- | --- |
| 2) | Configure the interface G0/1 on S1 as a trunk use the “switchport mode trunk” command |
| 3) | Give the “no shutdown” command for the subinterface G0/1.10 |
| 4) | Remove the incorrect Encapsulation dot1q, and assign the correct Encapsulation dot1q 10, then reassign the ip address for Subinterface G0/1.10 |
| 5) | Remove the incorrect Encapsulation dot1q, and assign the correct Encapsulation dot1q 30, then reassign the ip address for Subinterface G0/1.30 |

## Part 3: Verify Network Connectivity

Verify the PCs can ping each other and R1. If not, continue to troubleshoot until the pings are successful.

| PC1 ping PC3 |  |
| --- | --- |
| PC1 ping G0/1.10 on R1 |  |
| PC1 ping G0/1.30 on R1 |  |