# Lab 6.2.2.4 Packet Tracer – Configuring Basic EIGRP with IPv4

Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| R1 | G0/0 | 172.16.1.1 | 255.255.255.0 | N/A |
|  | S0/0/0 | 172.16.3.1 | 255.255.255.252 | N/A |
|  | S0/0/1 | 192.168.10.5 | 255.255.255.252 | N/A |
| R2 | G0/0 | 172.16.2.1 | 255.255.255.0 | N/A |
|  | S0/0/0 | 172.16.3.2 | 255.255.255.252 | N/A |
|  | S0/0/1 | 192.168.10.9 | 255.255.255.252 | N/A |
| R3 | G0/0 | 192.168.1.1 | 255.255.255.0 | N/A |
|  | S0/0/0 | 192.168.10.6 | 255.255.255.252 | N/A |
|  | S0/0/1 | 192.168.10.10 | 255.255.255.252 | N/A |
| PC1 | NIC | 172.16.1.10 | 255.255.255.0 | 172.16.1.1 |
| PC2 | NIC | 172.16.2.10 | 255.255.255.0 | 172.16.2.1 |
| PC3 | NIC | 192.168.1.10 | 255.255.255.0 | 192.168.1.1 |

Objectives

- Part 1: Configure EIGRP

- Part 2: Verify EIGRP Routing

Background

In this activity, you will implement basic EIGRP configurations including network commands, passive interfaces and disabling automatic summarization. You will then verify your EIGRP configuration using a variety of show commands and testing end-to-end connectivity.

## Part 1: Configure EIGRP

### Step 1: Enable the EIGRP routing process.

Enable the EIGRP routing process on each router using AS number 1. The configuration for R1 is shown.

R1(config)# router eigrp 1

What is the range of numbers that can be used for AS numbers?

1 to 65535

Note: Packet Tracer currently does not support the configuration of an EIGRP router ID.

### Step 2: Advertise directly connected networks.

- Use the show ip route command to display the directly connected networks on each router.

| R1 |  |
| --- | --- |
| R2 |  |
| R3 |  |

How can you tell the difference between subnet addresses and interface addresses?

C is subnet address L is interface address

- On each router, configure EIGRP to advertise the specific directly connected subnets. The configuration for R1 is shown.

R1(config-router)# network 172.16.1.0 0.0.0.255

R1(config-router)# network 172.16.3.0 0.0.0.3

R1(config-router)# network 192.168.10.4 0.0.0.3

| R1 | R1(config)#router eigrp 1 R1(config-router)#network 172.16.1.0 0.0.0.255 R1(config-router)#network 172.16.3.0 0.0.0.3 R1(config-router)#network 192.168.10.4 0.0.0.3 |
| --- | --- |
| R2 | R2(config)#router eigrp 1 R2(config-router)#network 172.16.2.0 0.0.0.255 R2(config-router)#network 172.16.3.0 0.0.0.3 R2(config-router)#network 192.168.10.8 0.0.0.3 |
| R3 | R3(config)#router eigrp 1 R3(config-router)#network 192.168.1.0 0.0.0.255 R3(config-router)#network 192.168.10.4 0.0.0.3 R3(config-router)#network 192.168.10.8 0.0.0.3 |

### Step 3: Configure passive interfaces.

Configure the LAN interfaces to not advertise EIGRP updates. The configuration for R1 is shown.

R1(config-router)# passive-interface g0/0

### Step 4: Disable automatic summarization.

The topology contains discontiguous networks. Therefore, disable automatic summarization on each router. The configuration for R1 is shown.

R1(config-router)# no auto-summary

Note: Prior to IOS 15 auto-summary had to be manually disabled.

### Step 5: Save the configurations.

| R1 |  |
| --- | --- |
| R2 |  |
| R3 |  |

## Part 2: Verify EIGRP Routing

### Step 1: Examine neighbor adjacencies.

- Which command displays the neighbors discovered by EIGRP?

show ip EIGRP neighbors

- All three routers should have two neighbors listed. The output for R1 should look similar to the following:

### Step 2: Display the EIGRP routing protocol parameters.

- What command displays the parameters and other information about the current state of any active IPv4 routing protocol processes configured on the router?

show ip protocol

- On R2, enter the command you listed for 2a and answer the following questions:

How many routers are sharing routing information with R2?

2 routers

Where is this information located under?

Routing Information Sources

What is the maximum hop count?

100

### Step 3: Verify end-to-end connectivity

PC1, PC2 and PC3 should now be able to ping each other. If not, troubleshoot your EIGRP configurations.

| PC1 ping PC2 | Successfully |
| --- | --- |
| PC1 ping PC3 | Successfully |
| PC2 ping PC3 | Successfully |