Lab - Research Computer Components

Use the Internet, trade publications, or a local store to gather information about the components you will need to complete your customer’s computer. Information is provided for the components that your customer already has. Use these specifications to make sure that the components you research are compatible with the components your customer already owns. Be prepared to discuss your selections.

  - Answer the following PC component questions.

    - List three components that must have the same compatible form factor.

Case, power supply, motherboard

    - List three components that must conform to the same socket type.

Fan,Motherboard, CPU

    - List two components that must utilize the same front side bus speed.

Motherboard, CPU

    - List three considerations when you choose memory.

number of pins, speed, dual channel capability

    - What component must be compatible with every other component of the computer?

Motherboard

  - Perform research for compatible components for your customer.

Your customer already owns the components described in the table below.

| Component | Brand and Model Number | Features | Cost |
| --- | --- | --- | --- |
| Case | NZXT Source 210 | ATX Mid Tower 2 x USB 2.0 Front Ports Bottom Mounted PSU Wire Management Support Dual 120mm Front Intake Also fits 2 x 120/140mm fan | $39.99 |
| Motherboard | ASUS H97-PRO Gamer | LGA 1150 Core i7 / i5 / i3 Intel H97 Chipset DDR3 1600/1333 MHz Non-ECC, Un-buffered 32GB Maximum Dual Channel Memory Supported 1 x PCIe 3.0/2.0 x16 1 x PCIe 2.0 x16 2 x PCI Express x1 4 x SATA 6Gb/s 4 x USB 3.0 | $104.99 |
| Hard Drive | Western Digital WD5000AAKX | SATA 6.0Gb/s 500GB 7200 RPM 16MB Cache | $52.99 |

Search the Internet, trade publications, or a local store to research the following components, make sure they are compatible with the components that your customer owns. Enter the specifications in the table below.

| Component | Brand and Model Number | Features | Cost |
| --- | --- | --- | --- |
| Power Supply | Rosewill ARC-450 | 450 Watt 20/24 pin 80 PLUS Bronze Certified 6 + 2-Pin PCI-E 120mm fan Active PFC | $44.99 |
| CPU | Intel i3-4160 Haswell | LGA 1150 Dual-Core 3.6GHz operating frequency 1066/1333MHz front side bus 2 X 256KB L2 cache 3 MB shared L3 cache 64 bit | $119.99 |
| Heatsink/Fan | ARCTIC Freezer i11 | LGA 2011 / 1150 / 1155 / 1156 92mm PWM Fan 74 CFM 22.5 dBA | $22.99 |
| RAM | G.SKILL Ripjaws Series F3-12800CL9D-4GBRL | 240 pin DDR3 1600 (PC3 12800) Cas Latency 9 DDR3 1333 (PC3 10666) Timing 9-9-9-24-2N Voltage 1.5V | $33.99 |
| Video Adaptor Card | EVGA GeForce GT 740 Superclocked | PCI Express 3.0 1059MHz Core Clock 2GB 128-Bit DDR3 1782MHz memory clock 128-Bit memory interface | $89.99 |