Lab - Install Adapter Cards

- Introduction

In this lab, you will install a NIC, a wireless NIC, and a video adapter card.

- Recommended Equipment

- Computer with power supply, motherboard, and drives installed

- NIC

- Wireless NIC

- Video adapter card

- Adapter card screws

- Antistatic wrist strap and antistatic mat

- Tool kit

- Motherboard manual

  - Install the wired NIC.

    - What type of expansion slot is compatible with the NIC?

PCI or PCIe.

    - Locate a compatible expansion slot for the NIC on the motherboard.

    - Remove the slot cover from the back of the case, if necessary.

    - Align the NIC to the expansion slot.

    - Press down gently on the NIC until the card is fully seated.

    - Secure the NIC by attaching the PC mounting bracket to the case with a screw.

  - Install the wireless NIC.

    - What type of expansion slot is compatible with the wireless NIC?

PCI or PCIe.

    - Locate a compatible expansion slot for the wireless NIC on the motherboard.

    - Remove the slot cover from the back of the case, if necessary.

    - Align the wireless NIC to the expansion slot.

    - Press down gently on the wireless NIC until the card is fully seated.

    - Secure the wireless NIC by attaching the PC mounting bracket to the case with a screw.

  - Install the video adapter card.

    - What type of expansion slot is compatible with the video adapter card?

PCI, AGP, or PCIe.

    - Locate a compatible expansion slot for the video adapter card on the motherboard.

    - Remove the slot cover(s) from the back of the case, if necessary.

    - Align the video adapter card to the expansion slot.

    - Press down gently on the video adapter card until the card is fully seated.

    - Secure the video adapter card by attaching the PC mounting bracket(s) to the case with a screw.

This lab is complete. Please have the instructor verify your work.